# Exterior Painting Checklist

| Job |  | Date |  |
| --- | --- | --- | --- |
| Address |  | Crew lead |  |

# Conditions gate

| Item | Done | Initials |
| --- | --- | --- |
| Forecast checked for the full coating and cure window, not only for the working hours |  |  |
| Surface temperature checked on each elevation, in shade and in sun, against the product range |  |  |
| Dew point checked and a stop time set that leaves the film out of its vulnerable stage by evening |  |  |
| Wind assessed for spray work, and masking increased or spraying deferred accordingly |  |  |

# Survey and protection

| Item | Done | Initials |
| --- | --- | --- |
| Full elevation walk done with a scraper, and failure extent compared against the estimate |  |  |
| Moisture sources identified and reported, rather than coated over |  |  |
| Change order raised and signed before any additional scope goes in |  |  |
| Plants, paths, driveways, furniture, vehicles and neighboring property protected |  |  |
| Windows, fittings, lights and meters masked, and doors sealed against dust |  |  |
| Ladder and access equipment set on firm level ground, tied or footed, checked each morning |  |  |

# Prep

| Item | Done | Initials |
| --- | --- | --- |
| House washed, then given genuine drying time before anything is scraped |  |  |
| Loose and failing coating scraped back to a sound edge, then feathered rather than left as a ridge |  |  |
| Biological growth treated rather than only washed off |  |  |
| Bare wood, filler and knots primed with the correct primer for the substrate |  |  |
| Exterior grade caulk applied after scraping and sanding, and tooled into the joint |  |  |

# Coating

| Item | Done | Initials |
| --- | --- | --- |
| Spread rate checked against the product data rather than judged by eye |  |  |
| Sprayed coats back rolled or back brushed on porous substrates |  |  |
| Work sequenced to follow the shade around the building through the day |  |  |
| Coats applied top down, body before trim, doors and spray work last |  |  |
| Recoat times respected as stated by the manufacturer for the actual conditions on site |  |  |

# Inspection and close

| Item | Done | Initials |
| --- | --- | --- |
| Each elevation inspected from ground level and from the ladder, in daylight, before moving on |  |  |
| Masking removed the same day, before it bonds or blows away overnight |  |  |
| Site cleared each evening, ladders taken down or secured, gates and access left as found |  |  |
| Leftover product labeled by elevation and color and left with the customer |  |  |

PaintPricing.com free template. Edit freely for your own jobs.