# Painting Quality Control Checklist

| Job |  | Date |  |
| --- | --- | --- | --- |
| Address |  | Crew lead |  |

# Hold point 1: after prep, before primer

| Item | Done | Initials |
| --- | --- | --- |
| Surfaces washed where required, and confirmed dry before any filler goes on |  |  |
| Filled areas checked by hand for proud edges, not judged by eye from across the room |  |  |
| Repairs sanded flush and feathered, with no visible ring around the patch |  |  |
| Sanding dust vacuumed and removed, not brushed onto the floor or into the corners |  |  |
| Glossy and previously coated surfaces scuff sanded properly rather than lightly scuffed |  |  |
| Caulk applied after sanding, tooled into the joint, and free of smears on the face |  |  |
| Masking lines checked for adhesion along the full run before any coating starts |  |  |

# Hold point 2: after primer, before the first coat

| Item | Done | Initials |
| --- | --- | --- |
| Primer applied where specified, and stains checked for bleed once it has dried |  |  |
| Patches confirmed invisible under primer before the first finish coat is applied |  |  |
| Color, sheen and location checked against the signed record at the point of opening the can |  |  |

# Hold point 3: after the first finish coat

| Item | Done | Initials |
| --- | --- | --- |
| First coat inspected for stipple, lap marks, ropes and heavy cut lines while a second coat remains |  |  |
| Recoat time respected against the product data for the actual conditions in the room |  |  |

# Hold point 4: after the final coat, before tape comes off

| Item | Done | Initials |
| --- | --- | --- |
| Cut lines checked for straightness and for the picture frame effect against the rolled field |  |  |
| Final coat inspected in raking light in every room, with a work light held along the surface |  |  |
| Ceilings checked from two positions, since a single viewing angle hides missed areas |  |  |
| Tape pulled at the correct stage and every line inspected for bleed or tearing |  |  |
| Trim, frames and doors checked for runs, sags and unpainted edges including the tops |  |  |

# Hold point 5: before the customer walk

| Item | Done | Initials |
| --- | --- | --- |
| Hardware, plates and vents refitted correctly, in the right rooms, with no paint on them |  |  |
| Protection removed only after the final inspection, never before |  |  |
| Independent walk done by someone who did not paint the room |  |  |
| Anything found recorded, so the pattern is visible rather than fixed and forgotten |  |  |
| Customer walk booked, and conducted with the lighting the customer actually uses |  |  |

# Defect record

| Room | Defect | Found by | Hold point |
| --- | --- | --- | --- |

PaintPricing.com free template. Edit freely for your own jobs.